| World Trade Organization |  |
| --- | --- |
|  | G/TBT/N/USA/507/Add.1 26 January 2011 |
|  | (11-0428) |
| Committee on Technical Barriers to Trade | Original: English |

notification

Addendum

The following communication, dated 20 January 2011, is being circulated at the request of the delegation of the United States.

_______________

TITLE:  Federal Motor Vehicle Safety Standards, Ejection Mitigation;  Phase-In Reporting Requirements; Incorporation by Reference

AGENCY:  National Highway Traffic Safety Administration (NHTSA), U.S. Department of Transportation (DOT)

ACTION:  Final rule

SUMMARY:  This final rule establishes a new Federal Motor Vehicle Safety Standard No. 226, "Ejection Mitigation'', to reduce the partial and complete ejection of vehicle occupants through side windows in crashes, particularly rollover crashes.  The standard applies to the side windows next to the first three rows of seats, and to a portion of the cargo area behind the first or second rows, in motor vehicles with a gross vehicle weight rating (GVWR) of 4,536 kilogram (kg) or less (10,000 pounds (lb) or less).  To assess compliance, the agency is adopting a test in which an impactor is propelled from inside a test vehicle toward the windows.  The ejection mitigation safety system is required to prevent the impactor from moving more than a specified distance beyond the plane of a window.  To ensure that the systems cover the entire opening of each window for the duration of a rollover, each side window will be impacted at up to four locations around its perimeter at two time intervals following deployment.

The agency anticipates that manufacturers will meet the standard by modifying existing side impact air bag curtains, and possibly supplementing them with advanced glazing.  The curtains will be made larger so that they cover more of the window opening, made more robust to remain inflated longer, and made to deploy in both side impacts and in rollovers.  In addition, after deployment the curtains will be tethered near the base of the vehicle's pillars or otherwise designed to keep the impactor within the boundaries established by the performance test.  This final rule adopts a phase-in of the new requirements, starting 1 September 2013.

This final rule advances NHTSA's initiatives in rollover safety and also responds to Section 10301 of the Safe, Accountable, Flexible, Efficient Transportation Equity Act: A Legacy for Users (SAFETEA-LU).  That section directs NHTSA to initiate and complete rulemaking to reduce complete and partial ejections of vehicle occupants from outboard seating positions, considering various ejection mitigation systems.

DATES:  Effective date:  The date on which this final rule amends the Code of Federal Regulations (CFR) is 1 March 2011.  The incorporation by reference of certain publications listed in the standard is approved by the Director of the Federal Register as of 1 March 2011.

Petitions for reconsideration:  If you wish to petition for reconsideration of this rule, your petition must be received by 7 March 2011.

Compliance dates:  This final rule adopts a phase-in of the new requirements.  The phase-in begins on 1 September 2013.  By 1 September 2017, all vehicles must meet the standard, with the exception of altered vehicles and vehicles produced in more than one stage, which are provided more time to meet the requirements.  Manufacturers can earn credits toward meeting the applicable phase-in percentages by producing compliant vehicles ahead of schedule, beginning 1 March 2011 and ending at the conclusion of the phase-in.

FULL TEXT URLs:

http://edocket.access.gpo.gov/2011/2011-547.htm

http://edocket.access.gpo.gov/2011/pdf/2011-547.pdf

__________