NOTIFICATION

The following notification is being circulated in accordance with Article 10.6

| 1. | Notifying Member: Switzerland If applicable, name of local government involved (Article 3.2 and 7.2): |
| --- | --- |
| 2. | Agency responsible: Federal Office of Communications Name and address (including telephone and fax numbers, email and website addresses, if available) of agency or authority designated to handle comments regarding the notification shall be indicated if different from above: State Secretariat for Economic Affairs SECO Holzikofenweg 36, 3003 Bern tbt@seco.admin.ch, www.seco.admin.ch |
| 3. | Notified under Article 2.9.2 [X], 2.10.1 [ ], 5.6.2 [ ], 5.7.1 [ ], other: |
| 4. | Products covered (HS or CCCN where applicable, otherwise national tariff heading. ICS numbers may be provided in addition, where applicable): Telecommunication equipment, radio equipment and telecommunication terminal equipment |
| 5. | Title, number of pages and language(s) of the notified document: Draft revision of the Ordinance of the Swiss Federal Office of Communications on telecommunications installations (OOIT) (2 page(s), in German; 2 page(s), in French; 2 page(s), in Italian) |
| 6. | Description of content: The Ordinance of the Swiss Federal Office of Communications on telecommunications installations (OOIT) specifies the basic technical requirements with regard to telecommunications installations set out by the Swiss Federal Council. It regulates the use of radio spectrum in Switzerland. A RIR for GBAS (Ground Based Augmentation System) will be introduced (RIR0102-07). Another RIR for ASDE (Airport Surface Detection Equipment) in the Frequency Range 9000 – 9500 MHz will be introduced (RIR0103-07). For the already existing RIR (RIR1003-07, RIR1003-08) the possibility of alternative mitigation techniques with equivalent effect will be mentioned due to changes in the ERC/REC 70-03. For data networks two RIR will be introduced in the frequency range 917.3 – 917.7 MHz according the ERC/REC 70-03 (RIR1003-13, RIR1003-14). In order to protect E-GSM-R the frequency Range 918.5 – 918.9 MHz, which is also foreseen by the ERC/REC 70-03, will not be usable. For person detection and collision avoidance one RIR in the frequency range 442.2 – 450 kHz will be introduced according to the ERC/REC 70-03 (RIR1003-15). For ULP-WMCE (Ultra Low Power Wireless Medical Capsule Endoscopy) one RIR in the frequency range 430 – 440 MHz will be introduced according to the ERC/REC 70-03 (RIR1006-12). For Non-specific SRDs one RIR in the frequency range 862 – 863 MHz will be introduced according to the ERC/REC 70-03 (RIR1008-43). The frequency range of the already existing RIR1010-07 will be extended from 57 - 66 GHz to 57 – 71 GHz according the ERC/REC 70-03. Another two RIR for wideband data transmission systems in the same frequency band will be introduced; one for fixed outdoor installations only and one for in- and outdoor applications according the ERC/REC 70-03 (RIR1010-09, RIR1010-10). |
| 7. | Objective and rationale, including the nature of urgent problems where applicable: The OOIT has to be updated to follow the latest frequency management in Europe.; Harmonization |
| 8. | Relevant documents: Link to all draft Radio Interface Regulations (RIR): https://www.ofcomnet.ch/#/rir/drafts Ordinance of 26th May 2016 of the Swiss Federal Office of Communications on telecommunications installations (OOIT; RS 784.101.21) - French: https://www.admin.ch/opc/fr/classified-compilation/20160601/index.html - German: https://www.admin.ch/opc/de/classified-compilation/20160601/index.html - Italian: https://www.admin.ch/opc/it/classified-compilation/20160601/index.html |
| 9. | Proposed date of adoption: 1 May 2020 Proposed date of entry into force: 1 May 2020 |
| 10. | Final date for comments: 20 April 2020 |
| 11. | Texts available from: National enquiry point [X] or address, telephone and fax numbers and email and website addresses, if available, of other body: https://members.wto.org/crnattachments/2020/TBT/CHE/20_0483_00_f.pdf https://members.wto.org/crnattachments/2020/TBT/CHE/20_0483_00_x1.pdf https://members.wto.org/crnattachments/2020/TBT/CHE/20_0483_00_x2.pdf |