| World Trade Organization |  |
| --- | --- |
|  | G/TBT/N/USA/138/Add.3 18 May 2009 |
|  | (09-2417) |
| Committee on Technical Barriers to Trade | Original: English |

notification

Addendum

The following communication, dated 13 May 2009, is being circulated at the request of the delegation of the United States.

_______________

TITLE: Federal Motor Vehicle Safety Standards; Roof Crush Resistance; Phase-In Reporting Requirements; Final Rule

AGENCY: National Highway Traffic Safety Administration (NHTSA), Department of Transportation

ACTION:  Final Rule

SUMMARY:  This Final rule upgrades the safety standard on roof crush resistance. First, for the vehicles currently subject to the standard, i.e., passenger cars and multipurpose passenger vehicles, trucks and buses with a Gross Vehicle Weight Rating (GVWR) of 2,722 kilograms (6,000 pounds) or less, the rule doubles the amount of force the vehicle's roof structure must withstand in the specified test, from 1.5 times the vehicle's unloaded weight to 3.0 times the vehicle's unloaded weight. Second, the rule extends the applicability of the standard so that it will also apply to vehicles with a GVWR greater than 2,722 kilograms (6,000 pounds), but not greater than 4,536 kilograms (10,000 pounds). The rule establishes a force requirement of 1.5 times the vehicle's unloaded weight for these newly included vehicles. Third, the rule requires all of the above vehicles to meet the specified force requirements in a two-sided test, instead of a single-sided test, i.e., the same vehicle must meet the force requirements when tested first on one side and then on the other side of the vehicle. Fourth, the rule establishes a new requirement for maintenance of headroom, i.e., survival space, during testing in addition to the existing limit on the amount of roof crush. The rule also includes a number of special provisions, including ones related to leadtime, to address the needs of multi-stage manufacturers, alterers, and small volume manufacturers.

Effective date: The date on which this final rule amends the CFR is 13 July 2009. The incorporation by reference of a publication listed in the rule is approved by the Director of the Federal Register as of 13 July 2009.

Compliance dates: Passenger cars and multipurpose passenger vehicles, trucks and buses with a GVWR of 2,722 kilograms (6,000 pounds) or less. This final rule adopts a phase-in of the upgraded roof crush resistance requirements for these vehicles. The phase-in begins on 1 September 2012. 
By 1 September 2015, all of these vehicles must meet the upgraded requirements, with certain exceptions. Vehicles produced in more than one stage and altered vehicles need not meet the upgraded requirements until 1 September 2016. Multipurpose passenger vehicles, trucks and buses with a GVWR greater than 2,722 kilograms (6,000 pounds) and less than or equal to 4,536 kilograms (10,000 pounds). All of these vehicles must meet the requirements beginning 1 September 2016, with certain exceptions. Vehicles produced in more than one stage and altered vehicles need not meet the requirements until 1 September 2017.

For further information contact:

Christopher J. Wiacek

NHTSA

Office of Crashworthiness Standards

1200 New Jersey Avenue, SE

West Building

Washington, DC 2059 USA

Tel: 202 366 4801

Full text at:

http://edocket.access.gpo.gov/2009/E9-10431.htm

http://edocket.access.gpo.gov/2009/pdf/E9-10431.pdf

__________