| World Trade Organization |  |
| --- | --- |
|  | G/TBT/N/USA/196/Corr.1 28 June 2006 |
|  | (06-3129) |
| Committee on Technical Barriers to Trade | Original: English |

- NOTIFICATION

- Corrigendum

The following communication, dated 26 June 2006, is being circulated at the request of the Delegation of the United States.

_______________

Standards of Performance for Stationary Spark Ignition Internal Combustion Engines and National Emission Standards for Hazardous Air Pollutants for Reciprocating Internal Combustion Engines

Correction

In proposed rule document 06-4919 beginning on page 33804 in the issue of Monday,  12 June 2006, make the following corrections:

          - On page 33808, in Table 1, in the table heading, in the first line, ``>19'' should read ``<=19''.

          - On page 33809, in Table 3, in the second column, in the third entry, ``HP<=500'' should read ``HP>=500''.

          - On the same page, in the same table, in the same column, in the fifth entry, ``HP>=500'' should read ``HP<500''.

          - On pages 33810 and 33811, Table 4 is being reprinted in its entirety to read as follows:

Table 4.--Emission Standards for Stationary RICE < =500 HP Located at Major Sources of HAP Emissions and Stationary RICE Located at Area Sources of HAP Emissions

Engine type and fuel  Maximum engine power Manufacture date a Emission standards

Existing All Fuels and All Types.  All Sizes No Emission Reduction.

New/Reconstructed SI < =25 HP

January 1, 2008 Meet 40 CFR part 60

SUBPART JJJJ.

New/Reconstructed SI Gasoline and  25< HP< 500  January 1, 2008  Meet 40 CFR part 60

Rich Burn LPG.HP>=500  July 1, 2007  subpart JJJJ.

New/Reconstructed Non-Emergency 25< HP< 500 a  January 1, 2008 1.0 g/HP-hr NMHC.

SI Natural Gas.

and

New/Reconstructed Non-Emergency	January 1, 2011		0.7 g/HP-hr NMHC.

SI Lean Burn LPG b.

New/Reconstructed Non-Emergency	HP>=500  July 1, 2007	1.0 g/HP-hr NMHC

SI Natural Gas.

and

New/Reconstructed Non-Emergency	July 1, 20100.7 g/HP-hr NMHC.

SI Lean Burn LPG.

New/Reconstructed Non-Emergency    250< HP < =500	January 1, 2008	93% CO Reduction

or

SI 4SLB at Major Sources (except 14 ppmvd  landfill and digester gas) b.formaldehyde.

CI All Fuels  All Sizes  2007+ Model Year  Meet 40 CFR part 60

SUBPART IIII.

Landfill/Digester Gas  HP< 500  January 1, 2008  1.0 g/HP-hr NMHC.

HP>=500  July 1, 2007  1.0 g/HP-hr NMHC.

Emergency SI  All Sizes  January 1, 2009  1.0 g/HP-hr NMHC.

            - Stationary SI natural gas and lean burn LPG engines between 19 and 37 KW (25 and 50 HP) may comply with the requirements of Table 2 of this preamble, instead of this table, as applicable.

            - New and reconstructed non-emergency 4SLB engines at major sources with a site rating between 250 and 500 HP are not required to meet the 1.0 and 0.7 g/HP-hr NMHC emission standards.

Table 3 to Subpart ZZZZ of Part 63 [Corrected]

          - On page 33844, in Table 3 to Subpart ZZZZ of Part 63, in the table heading, in the second line,``>500 HP'' should read ``<=500 HP''.

          - On page 33845, in the same table, in the table heading, in the second line,``>500 HP'' should read ``<=500 HP''.

Full text of the correction is online at URL:

http://a257.g.akamaitech.net/7/257/2422/01jan20061800/edocket.access.gpo.gov/2006/C6-4919.htm

__________