| World Trade Organization |  |
| --- | --- |
|  | G/TBT/N/USA/280 18 June 2007 |
|  | (07-2570) |
| Committee on Technical Barriers to Trade | Original: English |

NOTIFICATION

The following notification is being circulated in accordance with Article 10.6.

| 1. | Member to Agreement notifying: UNITED STATES If applicable, name of local government involved (Articles 3.2 and 7.2): |  |  |
| --- | --- | --- | --- |
| 2. | Agency responsible: Occupational Safety and Health Administration (OSHA) (290) Name and address (including telephone and fax numbers, e-mail and web-site addresses, if available) of agency or authority designated to handle comments regarding the notification shall be indicated if different from above: |  |  |
| 3. | Notified under Article 2.9.2 [ X ], 2.10.1 [ ], 5.6.2 [ ], 5.7.1 [ ], other: |  |  |
| 4. | Products covered (HS or CCCN where applicable, otherwise national tariff heading. ICS numbers may be provided in addition, where applicable): Power presses (ICS: 25.060, 25.080) |  |  |
| 5. | Title, number of pages and language(s) of the notified document: Power Presses (6 pages, in English) |  |  |
| 6. | Description of content: Mechanical power press safety is regulated under OSHA's mechanical power presses standard. OSHA adopted the standard in 1971, basing it upon the 1971 edition of American National Standards Institute (ANSI) B11.1, the industry consensus standard for mechanical power presses. This ANSI standard has been updated a number of times since OSHA adopted the 1971 version. The most recent edition was issued in 2001. Hydraulic and pneumatic power presses are not covered by OSHA's current standard. The original standard also did not address the use of presence-sensing-device initiation (PSDI) systems. When a press is equipped with PSDI, the press cycle will not initiate until the PSDI system senses that the danger zone is clear. OSHA updated the mechanical power presses standard on 14 March 1988, (53 FR 8353), to permit the use of PSDI systems. However, it requires an OSHA-approved third party to validate the PSDI system at installation and annually thereafter. Since the adoption of this provision, no third party has sought OSHA's approval. Consequently, PSDI systems are not being used with mechanical power presses. OSHA is seeking comments on whether and how the mechanical power presses standard should be amended, including whether the requirements pertaining to the use of PSDI systems should be revised and whether the scope of the standard should be expanded to cover other types of presses. |  |  |
| 7. | Objective and rationale, including the nature of urgent problems where applicable: Protection of human life |  |  |
| 8. | Relevant documents: 72 Federal Register (FR) 30729 4 June 2007; Title 29 Code of Federal Regulations (CFR) Part 1910. Will appear in the Federal Register when adopted. |  |  |
| 9. | Proposed date of adoption: Proposed date of entry into force: |  | To be determined |
| 10. | Final date for comments: 3 August 2007 |  |  |
| 11. | Texts available from: National enquiry point [ X ] or address, telephone and fax numbers, e-mail and web-site addresses, if available of the other body: Internet URLs: http://a257.g.akamaitech.net/7/257/2422/01jan20071800/edocket.access.gpo.gov/2007/E7-10655.htm http://a257.g.akamaitech.net/7/257/2422/01jan20071800/edocket.access.gpo.gov/2007/pdf/E7-10655.pdf |  |  |