| World Trade Organization |  |
| --- | --- |
|  | G/TBT/N/CHN/139 18 August 2005 |
|  | (05-3680) |
| Committee on Technical Barriers to Trade | Original: English |

NOTIFICATION

The following notification is being circulated in accordance with Article 10.6.

| 1. | Member to Agreement notifying: THE PEOPLE’S REPUBLIC OF CHINA If applicable, name of local government involved (Articles 3.2 and 7.2): |
| --- | --- |
| 2. | Agency responsible: Standardization Administration of China (SAC) Name and address (including telephone and fax numbers, e-mail and web-site addresses, if available) of agency or authority designated to handle comments regarding the notification shall be indicated if different from above: |
| 3. | Notified under Article 2.9.2 [ X ], 2.10.1 [ ], 5.6.2 [ ], 5.7.1 [ ], other: |
| 4. | Products covered (HS or CCCN where applicable, otherwise national tariff heading. ICS numbers may be provided in addition, where applicable): Electrolytic machine tools (ICS: 120.40; HS: 845690) |
| 5. | Title, number of pages and language(s) of the notified document: National Standard of the P.R.C., Electrolytic Machine Tools - Technical Requirement for Safeguarding (11 Pages, in Chinese) |
| 6. | Description of content: This standard specifies the safety requirements and measures, information for using, and the inspection of the safeguarding requirements for electrolytic machine tools. It applies to the equipments used in the process of electrochemical machining (ECM), such as electrochemical sinking machine (working voltage ≤ 60V DC), electrochemical polishing machine, electrochemical deburring machine, electrochemical cutting, electrochemical lathe, electrochemical grinding machine, electrochemical etching machine and other ECM machines. The safety requirements and measures of the above mentioned machines shall meet the following requirements: 1. Mechanical hazard and safeguarding 1.1 Impact hazard shall be prevented when emergency stop or power and /or control signal breaking off. 1.2 The cathode clamp of tools shall be safe and reliable. 1.3 The door/cover shall be interlocked with the electrolyte supply system, where there is electrolyte splashing. 1.4 Machining process shall be stopped or could not be started when the interlock of the safeguarding device is open. 1.5 The machine tools shall use double ram stroke limiting devices. 2. Safeguarding devices The removable safeguarding device shall be installed and dismantled easily. 3. Control system and devices 3.1 No hazard shall be occurred if the control signal carrier is broken off or damaged. 3.2 Error tolerance shall be designed for parts of control system that is easy to be mis-operated. 3.3 Emergency stop device shall be provided at each working station. 4. Electrical hazards and protection 4.1 Dual over-current protection device shall be provided for electrochemical sinking machine. 4.2 Anticorrosion measures shall be taken for naked conductor out of electrical cabinet. 4.3 Insulation between machine and workpiece shall be reliable. Insulating resistance shall be in accordance with 19.3 of GB 5226.1-2002. 4.4 The protection grade provided by the electrical cabinet shall be in accordance with 12.3 of GB 5226.1-2002. 5. Electrolyte system 5.1 The electrolyte circulation and filtration system shall not have any sign of leakage. Electrolyte shall not splash or drop out of the machine. 5.2 The electrolyte heating device and the heating measure taken shall not produce additional hazard. 5.3 The drainage of the electrolyte and its dregs shall be in accordance with relevant national rules for environmental protection. 6. Baneful gas prevention 6.1 Baneful gas and electrolyte fog produced during the processing shall be let off effectively in accordance with relevant national rules for environmental protection. 6.2 Checking and protecting devices for ventilation missing shall be equipped for machines in which mechanical ventilation measure is adopted. 7. Anti-corrosion protection 7.1 Anti-rust and anti-corrosion protection measure shall be taken for the important parts of the machine tools. 7.2 The surface of the bus bar shall be corrosion proofed. 7.3 Parts directly contacted with electrolyte shall be made of corrosion-proof material. 7.4 Parts of ventilation pipeline, fan and pipe connection shall be made of corrosion-proof material. 8. Noise 9. Lighting |
| 7. | Objective and rationale, including the nature of urgent problems where applicable: For the protection of human safety |
| 8. | Relevant documents: - |
| 9. | Proposed date of adoption: 90 days after circulation by WTO Secretariat Proposed date of entry into force: 6 months after adoption |
| 10. | Final date for comments: 60 days after circulation by WTO Secretariat |
| 11. | Texts available from: National enquiry point [ X ] or address, telephone and fax numbers, e-mail and web-site addresses, if available of the other body: WTO/TBT National Notification and Enquiry Center of the People’s Republic of China Tel: (+86) 10 82260618 Fax: (+86) 10 82262448 E-mail: tbt@aqsiq.gov.cn |