| World Trade Organization |  |
| --- | --- |
|  | G/TBT/N/USA/164/Add.1 6 August 2009 |
|  | (09-3813) |
| Committee on Technical Barriers to Trade | Original: English |

notification

Addendum

The following communication, dated 30 July 2009, is being circulated at the request of the delegation of United States.

_______________

TITLE:  Federal Motor Vehicle Safety Standards; Air Brake Systems; Final Rule

AGENCY:  Department of Transportation, National Highway Traffic Safety Administration

ACTION:  Final Rule

SUMMARY:  This document amends the Federal motor vehicle safety standard on air brake systems to improve the stopping distance performance of truck tractors. The rule requires the vast majority of new heavy truck tractors to achieve a 30 percent reduction in stopping distance compared to currently required levels. For these heavy truck tractors (approximately 99 percent of the fleet), the amended standard requires those vehicles to stop in not more than 250 feet when loaded to their gross vehicle weight rating (GVWR) and tested at a speed of 60 miles per hour (mph). For a small number of very heavy severe service tractors, the stopping distance requirement will be 310 feet under these same conditions. In addition, this final rule requires that all heavy truck tractors must stop within 235 feet when loaded to their lightly loaded vehicle weight  (LLVW).    The purpose of these amendments is to reduce the number of fatalities and injuries associated with crashes involving tractor-trailer combinations and other vehicles. In addition, we anticipate that this rule will prevent a substantial amount of property damage through averting or lessening the severity of crashes involving these vehicles. Once all subject heavy truck tractors on the road are equipped with enhanced braking systems, we estimate that annually, approximately 227 lives will be saved and 300 serious injuries will be prevented. In addition, this final rule is expected to prevent over $169 million in property damage annually, an amount which alone is expected to exceed the total cost of the rule.    There are a number of simple and effective manufacturing solutions that vehicle manufacturers can use to meet the requirements of this final rule. These solutions include installation of enhanced drum brakes, air disc brakes, or hybrid disc/drum systems. We note that currently a number of vehicles in the commercial fleet already utilize these improved braking systems and already realize performance that would meet the requirements of the amended standard. Includes responses to public review comments.

DATES:  Effective 24 November 2009.  Compliance Date: Three-axle tractors with a GVWR of 59,600 pounds or less must meet the reduced stopping distance requirements specified in this final rule by 1 August 2011. Two-axle tractors and tractors with a GVWR above 59,600 pounds must meet the reduced stopping distance requirements specified in this final rule by 1 August 2013.  Petitions for reconsideration must be received by NHTSA not later than 10 September 2009.

For further information contact:

Mr. Jeff Woods

Office of Crash Avoidance Standards

National Highway Traffic Safety Administration

U.S. Department of Transportation

1200 New Jersey Avenue, SE

Washington, DC 20590 USA

T:  202-366-6206

Fax: 202-366-7002

Email: jwoods@nhtsa.dot.gov

Full text at:

http://edocket.access.gpo.gov/2009/E9-17533.htm

http://edocket.access.gpo.gov/2009/pdf/E9-17533.pdf

__________