notification

Addendum

The following communication, dated 24 October 2016, is being circulated at the request of the delegation of the United States of America.

_______________

TITLE: Pipeline Safety: Expanding the Use of Excess Flow Valves in Gas Distribution Systems to Applications Other Than Single-Family Residences

AGENCY: Pipeline and Hazardous Materials Safety Administration (PHMSA), DOT

ACTION: Final rule

SUMMARY: Excess flow valves (EFV), which are safety devices installed on natural gas distribution pipelines to reduce the risk of accidents, are currently required for new or replaced gas service lines servicing single-family residences (SFR), as that phrase is defined in 49 CFR 192.383(a). This final rule makes changes to part 192 to expand this requirement to include new or replaced branched service lines servicing SFRs, multifamily residences, and small commercial entities consuming gas volumes not exceeding 1,000 Standard Cubic Feet per Hour (SCFH). PHMSA is also amending part 192 to require the use of either manual service line shut-off valves (e.g., curb valves) or EFVs, if appropriate, for new or replaced service lines with meter capacities exceeding 1,000 SCFH. Lastly, this final rule requires operators to notify customers of their right to request installation of an EFV on service lines that are not being newly installed or replaced. PHMSA has left the question of who bears the cost of installing EFVs on service lines not being newly installed or replaced to the operator's rate- setter.

DATES: This final rule is effective 14 April 2017.

https://members.wto.org/crnattachments/2016/TBT/USA/16_4439_00_e.pdf

__________