| World Trade Organization |  |
| --- | --- |
|  | G/TBT/N/SGP/6 14 November 2008 |
|  | (08-5550) |
| Committee on Technical Barriers to Trade | Original: English |

NOTIFICATION

The following notification is being circulated in accordance with Article 10.6

| 1. | Member to Agreement notifying: Singapore If applicable, name of local government involved (Article 3.2 and 7.2): |  |  |
| --- | --- | --- | --- |
| 2. | Agency responsible: Public Utilities Board Name and address (including telephone and fax numbers, email and web-site addresses, if available) of agency or authority designated to handle comments regarding the notification shall be indicated if different from above: |  |  |
| 3. | Notified under Article 2.9.2 [ X ], 2.10.1 [ ], 5.6.2 [ ], 5.7.1 [ ], Other: |  |  |
| 4. | Products covered (HS or CCCN where applicable, otherwise national tariff heading. ICS numbers may be provided in addition, where applicable): Please see Annex attached |  |  |
| 5. | Title, number of pages and language(s) of the notified document: Draft Public Utilities (Water Supply) (Amendment) Regulations 2008, 4 pages, English - Notice will be published in the Republic of Singapore’s Government Gazette when adopted. |  |  |
| 6. | Description of content: All products listed (namely, taps, flushing cisterns, and urinals) must be registered and labelled to provide information on their levels of water efficiency if they are to be supplied in Singapore. In order to register, the products must be tested to the test standards and requirements indicated in the Annex, which would be used to determine their water efficiency. For more details, please refer to PUB’s website – https://app.pub.gov.sg/wels/ |  |  |
| 7. | Objective and rationale, including the nature of urgent problems where applicable: To provide information on water efficiency of taps, flushing cisterns and urinals to consumers via labels. |  |  |
| 8. | Relevant documents: Draft Public Utilities (Water Supply) (Amendment) Regulations 2008, 4 pages, English - Notice will be published in the Republic of Singapore’s Government Gazette when adopted |  |  |
| 9. | Proposed date of adoption: Proposed date of entry into force: |  | 1 July 2009 |
| 10. | Final date for comments: 19 December 2008 |  |  |
| 11. | Text available from: National enquiry point [ ], or address, telephone and fax numbers, e-mail and web-site addresses, if available of the other body: Ramahad Singh Deputy Director Water Supply (Network) Department PUB Singapore Tel : (65) 6731 3406; Mobile: (65) 9730 3851 Fax : (65) 6731 3023 Email: ramahad_singhh@pub.gov.sg Lydia Loh Policy Executive Water Studies Division Ministry of the Environment and Water Resources 40 Scotts Road #10-01 Environment Building Singapore Republic of Singapore Tel.:(65) 6731 9193 Mobile: (65) 9710 6786 Fax: (65) 6731 9456 Email: lydia_loh@mewr.gov.sg |  |  |

Annex A

DETAILS OF RATINGS FOR WATER EFFICIENCY LABELLING

| Products/ Fittings | Test Standards and Requirements For Water Efficiency | Flow Rate/Flush Volume Requirements for Water Efficiency Labelling |
| --- | --- | --- |
| Shower Taps and Mixers (HS Codes: 84818051 84818059 84818091) | For Shower Taps (cold water only), Self-Closing-Delayed-Action Shower Taps, Sensor (including Self-Closing-Delayed-Action) Shower Taps or Combination Shower Tap Assemblies: Either of the following – Annex A of SS448 : Part 3 : 1998 or Section 10.3 of BS5412 : 1996 For mechanical shower mixers with a single or separate control device for adjusting flow rate and temperature: Section 10 of BS EN 817 : 1998 (with Constant Flow Regulators or restrictors and aerators attached. Tests to be conducted in increments of 0.5 bars from 1.0 to 3.0 bars) | Pressure Range from 1.0 to 3.0 bars Zero Tick : > 9 litres/min Good (1 Tick) : > 7 to 9 litres/min Very Good (2 Ticks) : > 5 to 7 litres/min Excellent (3 Ticks) : 5 litres/min or less |
| Basin Taps and Mixers (HS Codes: 84818051 84818059 84818091) | For Pillar Basin Taps, Self-Closing-Delayed-Action Basin Taps, Sensor (including Self-Closing-Delayed-Action) Basin Taps or Combination Basin Tap Assemblies: Either of the following – Annex A of SS448 : Part 3 : 1998 or Section 10.3 of BS5412 : 1996 For mechanical basin mixers with a single or separate control device for adjusting flow rate and temperature: Section 10 of BS EN 817 : 1998 (with Constant Flow Regulators or restrictors and aerators attached. Tests to be conducted in increments of 0.5 bars from 1.0 to 3.0 bars) | Pressure Range from 1.0 to 3.0 bars Zero Tick : > 6 litres/min Good (1 Tick) : > 4 to 6 litres/min Very Good (2 Ticks) : > 2 to 4 litres/min Excellent (3 Ticks) : 2 litres/min or less |
| Sink/Bib Taps and Mixers (HS Codes: 84818051 84818059 84818091) | For Bib Taps, Sink Taps (cold water only), Sensor Sink Taps or Combination Sink Tap Assemblies: Either of the following – Annex A of SS448 : Part 3 : 1998 or Section 10.3 of BS5412 : 1996 For mechanical sink mixers with a single or separate control device for adjusting flow rate and temperature: Section 10 of BS EN 817 : 1998 (with Constant Flow Regulators or restrictors and aerators attached. Tests to be conducted in increments of 0.5 bars from 1.0 to 3.0 bars) | Pressure Range from 1.0 to 3.0 bars Zero Tick : > 8 litres/min Good (1 Tick) : > 6 to 8 litres/min Very Good (2 Ticks) : > 4 to 6 litres/min Excellent (3 Ticks) : 4 litres/min or less |
| Flushing Cisterns (HS Codes: 39229019 69101000 69109000 73249010) | SS 378 : 1996 - Volume of Discharge per Flush (Dual Flush Low Capacity Flushing Cisterns - Full Flush & Reduced Flush) | Per Flush Good (1 Tick) : > 4 to 4.5 litres (full flush) & > 2.5 to 3 litres (reduced flush) Very Good (2 Ticks) : > 3.5 – 4.0 litres (full flush) & > 2.5 to 3 litres (reduced flush) Excellent (3 Ticks) : 3.5 litres or less(++) (full flush) & 2.5 litres or less (reduced flush) |
| Urinals and Urinal Flush Valves (HS Codes: 39229019 69101000 69109000 84818099 73249010) | Volume of Discharge Test (Please refer to PUB website at http://www.pub.gov.sg/general/Pages/FittingAndStandards.aspx) | Per Flush Good (1 Tick) : > 1 to 1.5 litres Very Good (2 Ticks) : > 0.5 to 1 litres Excellent (3 Ticks) : 0.5 litres or less(**) or Waterless Urinals |

Note:

++ To pass laboratory simulation test on waste transportation efficiency in pipes prescribed by PUB if full flush volume is less than 3.5 litres.

** To pass dilution tests prescribed by PUB if flush volume is less than 0.5 litres.

__________