| World Trade Organization |  |
| --- | --- |
|  | G/TBT/N/CHE/95 10 December 2007 |
|  | (07-5463) |
| Committee on Technical Barriers to Trade | Original: English |

NOTIFICATION

The following notification is being circulated in accordance with Article 10.6.

| 1. | Member to Agreement notifying: SWITZERLAND If applicable, name of local government involved (Articles 3.2 and 7.2): |  |  |
| --- | --- | --- | --- |
| 2. | Agency responsible: Federal Office for the Environment (FOEN) Air Pollution Control and NIR division Dr. Martin Schiess CH-3003 Berne Name and address (including telephone and fax numbers, e-mail and web-site addresses, if available) of agency or authority designated to handle comments regarding the notification shall be indicated if different from above: State Secretariat for Economic Affairs SECO Non-tariff Measures Division Effingerstrasse 1 CH-3003 Berne Tel: +(41) 31 323 25 95; Fax: +(41) 31 324 09 58; Email: kathrin.bucher@seco.admin.ch |  |  |
| 3. | Notified under Article 2.9.2 [ X ], 2.10.1 [ ], 5.6.2 [ ], 5.7.1 [ ], other: |  |  |
| 4. | Products covered (HS or CCCN where applicable, otherwise national tariff heading. ICS numbers may be provided in addition, where applicable): Diesel-powered machines and equipment for use on construction sites and similar installations |  |  |
| 5. | Title, number of pages and language(s) of the notified document: Draft revision of the Ordinance on Air Pollution Control (4 pages, in French and German). |  |  |
| 6. | Description of content: The draft ordinance contains measures to reduce particle emissions from diesel engines on construction sites. Most of these measures have already been in force since September 2003. The proposed amendment of the Ordinance on Air Pollution Control is due to the need for harmonized enforcement at the national level as well as the extension of the applicability of the measures to all construction sites. In particular, the draft ordinance stipulates that with effect from 1 May 2008 new diesel-powered construction machines over 37 kW as well as new machines between 18 and 37 kW from 2010 must comply with the following emission standards: - a new particle number emission limit value (1x1012 1/kWh according to the UNECE PMP-Method), - a limit ratio of NO2 to NOx (30 percent mass), - no use of copper-based additives or copper-based coatings in the exhaust system, - fulfilment of the requirements for the other pollutants at the time when the machines were put on the market, according to Directive 97/68/EG. These new emission standards are regarded as fulfilled, if the construction machines are equipped with tested and approved particulate filters. A certificate of conformity to the proposed regulation is mandatory for new diesel-powered machine types or for particle filter system types. A certificate of conformity must be issued by a recognized technical service or engine test laboratory. Also mandatory is labelling or marking of each individual diesel-powered machine indicating compliance with the new emission standards. |  |  |
| 7. | Objective and rationale, including the nature of urgent problems where applicable: Protection of human health: Airborne pollution in the form of particulate matter is currently one of the greatest challenges facing Swiss environmental policy. People living in cities and areas near major transport routes in particular are affected by high levels of particulate matter. In these areas the limit values of the Ordinance on Air Pollution Control are frequently exceeded, sometimes massively. Particularly urgent action is required in relation to diesel soot which is carcinogenic and must be minimized in accordance with environmental legislation as a precautionary measure. According to the WHO there is compelling toxicological evidence of deleterious health effects due to ultrafine particles. Diesel soot is emitted from engines into the atmosphere as a very large number of ultrafine particles with diameters of around 100 nanometers. In order to minimize diesel soot a particle number limit value is required in addition to the exhaust mass limit value. This principle, based on precautionary measures, is also implemented at the international level (UNECE-WP29-GRPE) by the PMP programme (Particle Measurement Programme). Highly effective particle filter systems with filtration efficiencies in excess of 97% (particle number) and a marked reduction in diesel soot (over 93%) are now state-of-the-art for construction machines and are also economically viable for large machines. |  |  |
| 8. | Relevant documents: Legal bases: Federal Law relating to the Protection of the Environment (LPE) http://www.admin.ch/ch/f/rs/c814_01.html Ordinance on Air Pollution Control (OPair) http://www.admin.ch/ch/f/rs/c814_318_142_1.html Air pollution Control at Constructions Sites: http://www.bafu.admin.ch/php/modules/shop/files/pdf/phpH2Wacn.pdf FOEN/Suva Filter List http://www.bafu.admin.ch/luft/00632/00639/00644/index.html?lang=en Particle Measurement Programme of the UNECE-WP.29-GRPE http://www.unece.org/trans/main/wp29/wp29wgs/wp29grpe/pmp19.html |  |  |
| 9. | Proposed date of adoption: Proposed date of entry into force: |  | April 2008 1 May 2008 |
| 10. | Final date for comments: 10 February 2008 |  |  |
| 11. | Texts available from: National enquiry point [ X ] or address, telephone and fax numbers, e-mail and web-site addresses, if available of the other body: http://www.bafu.admin.ch/aktuell/medieninformation/00004/index.html?lang=de&msg-id=16009 |  |  |