NOTIFICATION

The following notification is being circulated in accordance with Article 10.6

| 1. | Notifying Member: Israel If applicable, name of local government involved (Article 3.2 and 7.2): |  |  |
| --- | --- | --- | --- |
| 2. | Agency responsible: Israel WTO-TBT Enquiry Point Name and address (including telephone and fax numbers, email and website addresses, if available) of agency or authority designated to handle comments regarding the notification shall be indicated if different from above: Israel WTO-TBT Enquiry Point Ministry of Economy Tel.: + (972) 3 7347502 Fax: + (972) 3 7347626 E-mail: Yael.Friedgut@economy.gov.il |  |  |
| 3. | Notified under Article 2.9.2 [X], 2.10.1 [ ], 5.6.2 [ ], 5.7.1 [ ], other: |  |  |
| 4. | Products covered (HS or CCCN where applicable, otherwise national tariff heading. ICS numbers may be provided in addition, where applicable): Arc welding equipment; Welding equipment (ICS 25.160.30); Having a power handling capacity exceeding 1 kVA but not exceeding 16 kVA (HS 850432); - Machines and apparatus for arc (including plasma arc) welding of metals (HS 85153); - Parts (HS 851590). |  |  |
| 5. | Title, number of pages and language(s) of the notified document: Arc welding equipment: Limited duty equipment (74 pages in English, 6 pages in Hebrew). |  |  |
| 6. | Description of content: Revision of the mandatory standard SI 60974 part 6. This draft standard revision adopts the international standard IEC 60974-6 - Edition 2.0: 2010-11. The main significant technical changes with respect to the previous edition are the following: - Changes the touch current at fault condition to 7mA (paragraph 6.3.7); - Adds 2 new thermal requirements for two independent devices, one for thermal protection and one for thermal control (paragraph 7.1); - Adds a new requirement for thermal performances at 20°C (paragraph 7.2); - Adds a new requirement for an abnormal operation test for thermal control device (paragraph 10.4); - Introduces new requirements for auxiliaries, such as: wire feeder, torch, electrode holder and pressure regulator (paragraph 16); - Introduces a new requirement for a new rating plate definition and of a new mandatory warning symbols (paragraph 17). |  |  |
| 7. | Objective and rationale, including the nature of urgent problems where applicable: Protection of human health or Safety |  |  |
| 8. | Relevant documents: - Israel mandatory standard SI 60974 Part 6 (September 2010); - International standard IEC 60974-6 - Edition 2.0: 2010-11. |  |  |
| 9. | Proposed date of adoption: Proposed date of entry into force: |  | To be determined Generally 60 days after publication in Israel Official Gazette, Section of Government Notices |
| 10. | Final date for comments: 60 days from notification |  |  |
| 11. | Text available from: National enquiry point [X], or address, telephone and fax numbers, e-mail and web-site addresses, if available of the other body: http://www2.moital.gov.il/cmstamat/rsrc/tkina/SI60974Part6.pdf |  |  |