NOTIFICATION

The following notification is being circulated in accordance with Article 10.6

| 1. | Notifying Member: United States of America If applicable, name of local government involved (Article 3.2 and 7.2): |
| --- | --- |
| 2. | Agency responsible: Federal Aviation Administration (FAA) [961] Name and address (including telephone and fax numbers, email and website addresses, if available) of agency or authority designated to handle comments regarding the notification shall be indicated if different from above: Please submit comments to: USA WTO TBT Inquiry Point Email: ncsci@nist.gov |
| 3. | Notified under Article 2.9.2 [ X ], 2.10.1 [ ], 5.6.2 [ ], 5.7.1 [ ], other: |
| 4. | Products covered (HS or CCCN where applicable, otherwise national tariff heading. ICS numbers may be provided in addition, where applicable): Airplane fuel tanks. Accident and disaster control (ICS: 13.200), Explosion protection (ICS: 13.230), Aircraft and space vehicles in general (ICS: 49.020) |
| 5. | Title, number of pages and language(s) of the notified document: Transport Airplane Fuel Tank and System Lightning Protection (10 pages, in English) |
| 6. | Description of content: The FAA proposes to amend certain airworthiness regulations for transport category airplanes regarding lightning protection of fuel tanks and systems. This action would establish design requirements for both normal conditions and possible failures of fuel tank structure and systems that could lead to fuel tank explosions, add new maintenance requirements related to lightning protection features, and impose specific requirements for airworthiness limitations in the instructions for continued airworthiness. We would create performance-based standards for prevention of catastrophic fuel vapor ignition caused by lightning by regulating the risk due to both ignition sources and fuel tank flammability. This change would allow designers to take advantage of flammability reduction technologies whose effectiveness was not foreseen when earlier revisions to these rules were written. This change would also relieve some of the administrative burdens created by the current regulations. These proposed amendments are based on recommendations from the Large Airplane Fuel System Lightning Protection Aviation Rulemaking Committee (Lightning ARC). |
| 7. | Objective and rationale, including the nature of urgent problems where applicable: Protection of human health or safety |
| 8. | Relevant documents: 79 Federal Register (FR) 75496, 18 December 2014; Title 14 Code of Federal Regulations (CFR) Part 25. Will appear in the Federal Register when adopted. |
| 9. | Proposed date of adoption: To be determined Proposed date of entry into force: To be determined |
| 10. | Final date for comments: 18 March 2015 |
| 11. | Texts available from: National enquiry point [ ] or address, telephone and fax numbers and email and website addresses, if available, of other body: http://www.gpo.gov/fdsys/pkg/FR-2014-12-18/html/2014-29385.htm http://www.gpo.gov/fdsys/pkg/FR-2014-12-18/pdf/2014-29385.pdf |